# Node-Media-Server 2.7.5 + FFmpeg (DASH) — Next Steps Guide

Target: Windows (offline LAN) • Client: Android (DASH/ExoPlayer)

2026-02-06

# Step-by-step: what to do next

Quick checklist:

Make FFmpeg reachable from Node-Media-Server (path in server.js).

Start the server (RTMP 1935 + HTTP 8000).

Publish a test stream to RTMP.

Confirm index.mpd + segments appear under ./media.

Allow LAN access via Windows Firewall (if needed).

## 1) Decide where FFmpeg will live (recommended: inside the server folder)

Node-Media-Server calls FFmpeg to create DASH segments. The first thing is to ensure the FFmpeg path in server.js points to a real ffmpeg.exe.

Recommended folder structure

E:\New folder (2)\ Node-Media-Server-2.7.5\ server.js node_modules\ media\ (created by you / auto-created) ffmpeg\ bin\ ffmpeg.exe ffprobe.exe ffplay.exe (optional)

Two valid options:

Option A (recommended): copy your "ffmpeg-8.0.1" folder into "Node-Media-Server-2.7.5\ffmpeg" (so server.js can use a short relative path).

Option B: keep FFmpeg where it is now, and set an absolute path in server.js (example: E:/New folder (2)/ffmpeg-8.0.1/bin/ffmpeg.exe).

Tip: In JavaScript strings, using forward slashes (E:/path/to/ffmpeg.exe) avoids escaping backslashes.

## 2) Verify FFmpeg runs on this machine

Open Command Prompt (cmd.exe) and run:

cd /d E:\New folder (2)\Node-Media-Server-2.7.5 .\ffmpeg\bin\ffmpeg.exe -version

If you chose Option B (FFmpeg kept outside), run ffmpeg.exe using its real path (or add FFmpeg to PATH).

## 3) Check your server.js import (important!)

Because you downloaded the source code (not the npm package), you must import Node-Media-Server using a relative path.

Do this quick check:

Open package.json and look for the "main" entry (it usually points to something like src/node_media_server.js).

In server.js, require the module using one of these patterns:

// Pattern 1: load from the project root (uses package.json "main") const NodeMediaServer = require('./'); // Pattern 2: load directly from the main file // (update the path if your package.json "main" is different) const NodeMediaServer = require('./src/node_media_server');

If you accidentally used: require('node-media-server'), you may see "Cannot find module 'node-media-server'". Use the relative require shown above.

## 4) Set the FFmpeg path + enable DASH remux

Make sure your server.js config contains a transcode/remux section with dash enabled. Example configuration:

const NodeMediaServer = require('./'); const config = { logType: 3, rtmp: { port: 1935, chunk_size: 60000, gop_cache: true, ping: 30, ping_timeout: 60, }, http: { port: 8000, mediaroot: './media', allow_origin: '*', }, trans: { // Use a path that exists on your machine: // Option A (FFmpeg inside project): ffmpeg: './ffmpeg/bin/ffmpeg.exe', // Option B (FFmpeg outside project): // ffmpeg: 'E:/New folder (2)/ffmpeg-8.0.1/bin/ffmpeg.exe', tasks: [ { app: 'live', hls: true, hlsFlags: '[hls_time=2:hls_list_size=3:hls_flags=delete_segments]', dash: true, dashFlags: '[f=dash:window_size=3:extra_window_size=5]', }, ], }, // For first test, you can disable auth (simpler). // After streaming works, enable auth and implement your own checks. auth: { play: false, publish: false, secret: 'CHANGE_ME', }, }; const nms = new NodeMediaServer(config); nms.run();

Important details:

The RTMP publish URL must match the app name (app: 'live').

DASH output will be created under: .\media\live\<streamKey>\index.mpd

If you run with auth disabled first, you remove one big source of problems during initial testing.

## 5) Start the server and confirm it is listening

From the Node-Media-Server folder:

cd /d E:\New folder (2)\Node-Media-Server-2.7.5 node server.js

Expected result: console logs showing RTMP on port 1935 and HTTP on port 8000.

## 6) Publish a test stream (no OBS needed)

Use FFmpeg to generate a test video + audio and push via RTMP. Run this in a second cmd window:

cd /d E:\New folder (2)\Node-Media-Server-2.7.5 .\ffmpeg\bin\ffmpeg.exe -re ^ -f lavfi -i testsrc=size=1280x720:rate=30 ^ -f lavfi -i sine=frequency=1000 ^ -c:v libx264 -preset veryfast -tune zerolatency -g 60 -keyint_min 60 -b:v 1500k ^ -c:a aac -b:a 128k ^ -f flv rtmp://127.0.0.1/live/stream1

Notes:

stream1 is your stream key. You can change it to anything (letters/numbers).

If your server is on another machine, replace 127.0.0.1 with the server's LAN IP (example: 192.168.1.10).

## 7) Verify that DASH output is being generated

While the stream is publishing, check the output folder:

E:\New folder (2)\Node-Media-Server-2.7.5\media\live\stream1\

index.mpd init-stream*.m4s (or init*.mp4 depending on FFmpeg) chunk-stream*.m4s (segments)

If the folder is empty or index.mpd is missing, go to the Troubleshooting section below.

## 8) Test playback (quick checks)

The DASH manifest URL should be:

http://127.0.0.1:8000/live/stream1/index.mpd

Quick ways to test without writing an Android app yet:

If you have ffplay.exe: open it with the URL above.

If you have VLC: Media -> Open Network Stream -> paste the URL.

Best real-world test: play with your Android ExoPlayer app (Media3).

## 9) Allow LAN access (Windows Firewall)

If clients on other devices cannot connect, create inbound firewall rules for:

TCP 1935 (RTMP ingest)

TCP 8000 (HTTP for DASH segments + index.mpd)

PowerShell (run as Administrator) example:

New-NetFirewallRule -DisplayName "NMS RTMP 1935" -Direction Inbound -Action Allow -Protocol TCP -LocalPort 1935 New-NetFirewallRule -DisplayName "NMS HTTP 8000" -Direction Inbound -Action Allow -Protocol TCP -LocalPort 8000

# Troubleshooting

## A) "Cannot find module 'node-media-server'"

Cause: server.js uses require('node-media-server') but you cloned the source code (not installed from npm).

Fix: change to a relative import:

const NodeMediaServer = require('./');

## B) FFmpeg not found / ENOENT / "spawn ffmpeg" errors

Cause: the ffmpeg path in config.trans.ffmpeg is wrong.

Fix checklist:

Confirm the file exists (ffmpeg.exe).

Use forward slashes or double-backslashes in the path.

Run "ffmpeg.exe -version" from cmd to confirm it runs.

## C) Stream publishes but no DASH output is created

Fix checklist:

Confirm your publish URL uses the same app name as trans.tasks.app (example uses live).

Confirm the HTTP mediaroot is writable (./media).

Check the server console for FFmpeg errors (codec unsupported, permission errors, etc.).

Try publishing H.264 + AAC (recommended for maximum compatibility).

## D) Playback stutters or uses too much bandwidth

DASH only reduces bandwidth if you provide multiple renditions (adaptive bitrates) or if your source bitrate is already low.

Practical options:

Set a lower bitrate on the source device (best CPU efficiency).

Transcode on the server to multiple bitrates (higher CPU usage).

Increase segment duration to reduce request rate (trade-off: higher latency).

# After the first successful test: recommended next work

## 1) Add real authentication + stream key management

Once you confirm publish/play works, add your own logic:

Use prePublish to validate the publisher (device) and stream key.

Use prePlay to validate viewers (JWT / signed token / ACL rules).

Maintain a device registry (your decentralized sources).

## 2) Make it production-friendly on Windows

Suggested improvements:

Create start.bat to launch the server.

Run it as a background service (pm2 or NSSM).

Configure log rotation.

## 3) Package for offline deployment (copy to another Windows PC)

Goal: make a self-contained folder you can zip and copy to an offline machine.

A) Create a start script (start.bat) in the project root:

@echo off cd /d %~dp0 node server.js pause

B) Optional: remove dev-only dependencies to reduce size (run once):

cd /d E:\New folder (2)\Node-Media-Server-2.7.5 npm prune --omit=dev

C) Zip the folder (PowerShell example):

$src = "E:\New folder (2)\Node-Media-Server-2.7.5" $dst = "E:\Node-Media-Server-2.7.5_bundle.zip" Compress-Archive -Path $src -DestinationPath $dst -Force

D) On the offline PC: unzip, ensure Node.js is installed, then run start.bat.

Tip: if Node.js is NOT installed on the offline PC, you can also copy a portable Node distribution and change start.bat to call node.exe using a relative path.

## 4) Android client: play DASH with ExoPlayer (Media3)

Implement a minimal player that loads:

http://SERVER_IP:8000/live/<streamKey>/index.mpd

Then add login + token handling when you enable auth on the server.